CANopen Node Registry

Spreadsheet Guide for Subteam Leads

# Purpose

This workbook coordinates the CAN bus architecture across all subteams. Every board that communicates on the spacecraft's CAN bus needs a unique Node ID, and every signal that travels over that bus needs to be mapped to a PDO (Process Data Object). This document explains how to fill in each sheet correctly so that Avionics / C&DH can assign Node IDs and build the Object Dictionary without going back and forth with each team.

## Cell Colour Legend

Before filling in anything, understand what each cell colour means:

| Colour | Meaning | What to do |
| --- | --- | --- |
|  | Yellow | Fill this in — this is your input. |
|  | Light blue | Pre-filled reference data. Do not edit. |
|  | Grey | Calculated or assigned by Avionics / C&DH. Leave blank. |
|  | White | Pre-filled data row. Read only. |

# Sheet 1 — Node Registry

This is the master list of every board on the CAN bus. Each row represents one physical board (one microcontroller running firmware that participates in the network). Avionics will pre-fill the known nodes; subteams add their own.

Note: If your subsystem only has one board, you only add one row. If it has two independent microcontrollers that each need to talk on the bus, add two rows.

## Column-by-Column Guide

| Column | What to fill in | Example |
| --- | --- | --- |
| Node ID (hex) | Leave blank. Avionics assigns this after reviewing all requests. | 0x03 |
| Node ID (dec) | Auto-calculated. Do not touch. | 3 |
| Node Name | A short, descriptive name for the board. Use something that will be unambiguous in six months. | EPS Controller |
| Subteam Owner | Your subteam name. This is who Software will contact if there is an issue with this node. | EPS |
| Role (Master/Slave) | Almost always Slave. There is exactly one Master on the bus (OBC). Choose from the dropdown. | Slave |
| NMT State (target) | The NMT state your node should be in during normal operation. Almost always Operational. Choose from the dropdown. | Operational |
| HB Period (ms) | How often your node should broadcast a heartbeat. 500 ms is a reasonable default. If you have no preference, write 500. | 500 |
| TPDO Count | How many signals your board transmits onto the bus. Count the number of TX rows you will add in Sheet 2. | 2 |
| RPDO Count | How many signals your board receives from the bus. Count the number of RX rows you will add in Sheet 2. | 1 |
| SDO Server? | Whether the master should be able to read or write your node's Object Dictionary over SDO (for configuration or diagnostics). Almost always Yes. | Yes |
| SDO Client? | Whether your node initiates SDO requests to other nodes. Almost always No unless OBC specifically asks for this. | No |
| Notes / Purpose | A plain-English description of what this board does on the spacecraft. Be specific — 'sensor node' is not enough; 'monitors battery voltage, temperature, and charge current for the primary battery' is. | Monitors battery cell voltages, temperatures, and charge current for the primary battery pack. |

⚠  Do not assign your own Node ID. Two nodes with the same ID will corrupt each other's frames and bring down the entire bus. Leave column A blank and the software team will assign IDs once all requests are in.

## Common Mistakes on Sheet 1

- Putting one row per signal instead of one row per board. Sheet 1 is about boards, not signals. Signals go in Sheet 2.

- Leaving Node Name blank or using a generic name like 'Board 1'. The name appears in debug logs and Wireshark captures — make it meaningful.

- Setting HB Period to 0 or leaving it blank. Every node must produce a heartbeat, so the master knows it is alive.

- Setting TPDO / RPDO counts to 0 when the node has signals. If your board sends anything, TPDO count should be ≥ 1. If it receives anything, RPDO count should be ≥ 1. The maximum is 4 in each direction per CANopen standard.

# Sheet 2 — PDO & OD Map

This sheet maps every signal that travels on the CAN bus. Where Sheet 1 is about boards, Sheet 2 is about the data those boards exchange. One row = one signal.

The columns are split into two colour-coded groups that represent two different things:

| Group | What it describes | Who fills it in |
| --- | --- | --- |
| PDO Comm Parameters (blue columns G–J) | How the PDO behaves — its transmission type, period, and which fixed OD object (0x1xxx) configures it. These indices are defined by the CANopen standard and are not chosen by subteams. | Software fills these in after reviewing subteam requests. |
| Application Data Mapping (green columns K–N) | Where the signal's value lives in the Object Dictionary — the manufacturer-specific index (0x2xxx), subindex, data type, and variable name. You choose the index within the 0x2000–0x5FFF range. | Subteams fill in signal name, data type, and description. Software assigns the OD index. |

## Column-by-Column Guide

### Node columns (A–B)

| Column | What to fill in | Example |
| --- | --- | --- |
| Node ID (hex) | The hex Node ID of the board this signal belongs to. Copy from Sheet 1 once Software assigns it. Leave blank if not yet assigned. | 0x03 |
| Node Name | The name of the board, matching Sheet 1. | EPS Controller |

### PDO Identity columns (C–F)

These identify which PDO slot this signal occupies on the node.

| Column | What to fill in | Example |
| --- | --- | --- |
| PDO Label | A short name for the PDO frame this signal travels in. If two signals share the same PDO frame (they are packed together), they should have the same label. | TPDO1 |
| Direction (TX/RX) | TX if your node sends this signal. RX if your node receives it. Always from the perspective of the node in column A. | TX |
| PDO Number (1–4) | Which PDO slot this uses on the node — 1, 2, 3, or 4. Each node has at most 4 TX slots and 4 RX slots. If this is your node's first TX PDO, write 1. | 1 |
| COB-ID | Leave blank. Auto-calculated by software from the PDO number and Node ID using the formula: function code offset + Node ID. | 0x183 |

Note: PDO Number matters because it determines which fixed OD objects (0x1800, 0x1801 etc.) configure this PDO. PDO Number 1 always uses comm object 0x1800 for TX or 0x1400 for RX. PDO Number 2 uses 0x1801 / 0x1401, and so on.

### PDO Comm Parameter columns (G–J) — Blue

These are filled in by the software team based on your update rate requirements from Sheet 4. You do not need to fill these in, but understanding them helps you give better answers in the request form.

| Column | What it means |
| --- | --- |
| Comm Param OD Index (0x1xxx) | The CiA 301 fixed index that configures this PDO's behaviour. Not your choice — determined by PDO number. |
| Tx Type (OD 0x1800:02) | How the PDO is triggered: Cyclic (on a timer), Sync+Event (on SYNC pulse or value change), or Event (on change). |
| Period ms (OD 0x1800:05) | For cyclic PDOs, how often the frame is sent in milliseconds. |
| Inhibit ms (OD 0x1800:03) | For event-driven PDOs, the minimum gap between two consecutive transmissions of this PDO. Prevents a rapidly-toggling signal from flooding the bus. |

### Application Data Mapping columns (K–N) — Green

These are the most important columns for subteams to fill in. They describe the actual signal.

| Column | What to fill in | Example |
| --- | --- | --- |
| Signal / Variable Name | The exact name of the variable in your firmware code. Using the real variable name makes it much easier to verify the PDO mapping is wired correctly. | batteryVoltage_mV |
| App OD Index (0x2xxx) | Leave blank for now. Software team assigns an index in the 0x2000–0x5FFF range after reviewing all requests. This is the address of your signal in the Object Dictionary. | 0x2200 |
| App OD Subindex | Leave blank. Almost always 0x00 for a simple variable. Software team will fill this in. | 0x00 |
| Data Type | The C data type of this variable. Choose from the dropdown. Use the smallest type that fits your data range. | UINT16 |

### Choosing a Data Type

Use this as a reference when filling in the Data Type column:

| Type | C equivalent | Range | Bits | When to use |
| --- | --- | --- | --- | --- |
| UINT8 | uint8_t | 0 to 255 | 8 | Status flags, mode bytes, small counters |
| UINT16 | uint16_t | 0 to 65,535 | 16 | Voltages in mV, temperatures ×10, RPM |
| UINT32 | uint32_t | 0 to 4,294,967,295 | 32 | Timestamps, large counters, packed flags |
| INT8 | int8_t | -128 to 127 | 8 | Small signed values |
| INT16 | int16_t | -32,768 to 32,767 | 16 | Signed sensor values, setpoints |
| INT32 | int32_t | -2,147,483,648 to 2,147,483,647 | 32 | Large signed values |
| BOOL | uint8_t | 0 or 1 | 8 | Single true/false flags |
| REAL32 | float | IEEE 754 float | 32 | Avoid if possible — discuss with software first |

⚠  Do not use REAL32 (float) without first checking with the software team. CANopen does not natively define a float type and handling it requires a custom OD entry. It is almost always better to scale your value to an integer (e.g. store voltage as millivolts in a UINT16 rather than volts in a REAL32).

### Signal Detail columns (O–P)

| Column | What to fill in | Example |
| --- | --- | --- |
| Size (bits) | How many bits this signal takes in the PDO frame. Must match the data type: UINT8 = 8 bits, UINT16 = 16 bits, INT32 = 32 bits. One PDO frame can carry a maximum of 64 bits (8 bytes) total across all signals packed into it. | 16 |
| Description / Units | A plain-English description of the signal including physical units. This is critical — the receiving node needs to know how to interpret the raw value. | Battery pack voltage in millivolts. Range 0–25000 mV. |

## PDO Packing Rules

Multiple signals can travel in the same PDO frame, which is more efficient than one frame per signal. When packing signals into one PDO, the total size of all signals combined must not exceed 64 bits (8 bytes).

Example — three signals packed into one TPDO1 frame:

| Signal | Type | Bits | Running total |
| --- | --- | --- | --- |
| batteryVoltage_mV | UINT16 | 16 | 16 bits |
| batteryTemp_degC | INT16 | 16 | 32 bits |
| chargeState | UINT8 | 8 | 40 bits ✓ (under 64) |

Note: Signals packed into the same PDO frame share the same PDO Label and PDO Number in columns C and E. Their App OD Indices (column L) will each be different — each signal has its own place in the Object Dictionary even if they travel together in one frame.

# Sheet 3 — COB-ID Reference

This sheet is read-only. It is a reference table that shows every CANopen function code and the resulting COB-ID for each type of message. You do not fill anything in here.

A COB-ID (Communication Object Identifier) is the CAN frame ID that a message travels on. Every CANopen message type has a fixed function code offset. The COB-ID is always:

COB-ID  =  Function Code Offset  +  Node ID

For example, TPDO1 has a function code offset of 0x180. If your node is assigned Node ID 0x03, then your TPDO1 frame will have COB-ID 0x183 (0x180 + 0x03). This is why the software team needs to assign Node IDs centrally — two nodes with the same ID would produce COB-IDs that collide on the bus.

The table on Sheet 3 shows the COB-IDs for the two existing reference nodes (0x01 and 0x02) as concrete examples. Use it when you want to verify which CAN frame ID your signal will appear on, or when setting up a CAN analyser to filter for your node's traffic.

# Sheet 4 — Subteam Request Form

This is the primary sheet for subteams to fill in. It is designed to be completed without needing to understand CANopen internals. Describe your signals in plain terms and the software team will translate them into OD entries and PDO mappings in Sheet 2.

Note: Fill in one row per signal your subsystem needs to send or receive. If your board sends 3 signals and receives 1, you need 4 rows.

## Column-by-Column Guide

| Column | What to fill in | Example |
| --- | --- | --- |
| Subteam Name | Your subteam name. Fill this in on every row — it makes it easier for software to sort and group requests. | EPS |
| Node Name (proposed) | What you want to call your board. Use the same name on every row for signals from the same board. Software may rename it slightly for consistency. | EPS Power Monitor |
| Assigned Node ID | Leave blank. Grey cells are filled in by the software team after reviewing all submissions. |  |
| Signal / Variable Name | The exact name of the variable in your firmware. This should match your code so there is no ambiguity when wiring up the PDO mapping. | batteryVoltage_mV |
| TX or RX | TX if your node sends this signal. RX if your node receives it. Choose from the dropdown. | TX |
| Data Type | The C type of this variable. See the Data Type table in the Sheet 2 section above if you are unsure. Choose from the dropdown. | UINT16 |
| Update Rate / Trigger | How often or under what condition this signal should be sent. Be as specific as possible. See the guide below. | Every 100 ms |
| Description / Units | What this signal represents in plain English, including physical units and value range. The more detail here, the less back-and-forth with the software team. | Battery pack voltage. Units: millivolts. Range: 0–25000 mV. Normal operating range: 14000–16800 mV. |

## Filling in Update Rate / Trigger

This column is the most important one for Software to configure the PDO correctly. It determines the PDO transmission type. Give one of the following types of answer:

| What you write | What Software Team configures | When to use this |
| --- | --- | --- |
| Every X ms e.g. 'Every 100 ms' | Cyclic PDO — frame is sent on a fixed timer regardless of whether the value changed. | Telemetry, housekeeping data, anything that should be reported continuously. Most sensor readings. |
| On change or 'When value changes' | Event-driven PDO — frame is sent immediately when the value changes. Software will also set an inhibit time to prevent flooding. | Commands, alarms, mode changes, anything where latency matters more than regularity. |
| On SYNC or 'Synchronised with other nodes' | SYNC-based PDO — frame is sent once per SYNC pulse from the master. | When your signal needs to be time-correlated with signals from other nodes (e.g. attitude + position must be sampled at the same instant). |
| On request or 'Only when master asks' | RTR-triggered PDO or SDO read — master polls your node when it needs the value. | Slow-changing data that does not need to be broadcast continuously (e.g. configuration parameters, calibration values). |

⚠  Do not write 'as fast as possible' or 'continuously'. An uninhibited event-driven PDO from a noisy sensor can send hundreds of frames per second and saturate a 500 kbps bus in milliseconds. Give a concrete period or minimum interval even if it is an estimate.

## How Requests Flow to Sheets 1 and 2

Once all subteams return their completed Sheet 4, the software team will:

- Review all requests and identify any conflicts (two teams claiming the same signal name, unrealistic update rates, bus loading issues).

- Assign Node IDs on Sheet 1, filling in the grey 'Assigned Node ID' column on Sheet 4 to confirm.

- Assign App OD Indices (0x2xxx) on Sheet 2, one per signal.

- Configure PDO comm parameters (transmission type, period, inhibit time) based on your update rate answers.

- Distribute the finalised Sheet 2 back to all subteams so each team knows exactly which OD index and PDO number to use in their firmware.

# Frequently Asked Questions

## My subsystem has two boards. Do I add two rows in Sheet 1?

Yes — one row per board that independently participates in the CAN bus. If the two boards share a microcontroller or one only talks to the other over a different internal bus (SPI, I2C, UART), only the one that connects to CAN needs a row.

## What is the maximum number of signals my node can send?

A CANopen node has at most 4 TPDO slots and 4 RPDO slots. Each PDO frame carries up to 8 bytes (64 bits) of signal data. So the maximum is 4 × 8 = 32 bytes of TX data and 32 bytes of RX data per node per cycle. If you need more than that, discuss with the software team — one option is splitting your functionality across two nodes.

## Can two signals from different nodes share the same PDO?

No. A PDO belongs to exactly one node — the node whose COB-ID it uses. Signals from different nodes always travel in different frames. If two nodes need to exchange data, one sends a TPDO and the other receives it as an RPDO.

## What does the Comm Param OD Index column mean?

It identifies which fixed CiA 301 object configures this PDO's behaviour. These are not chosen by subteams — the CANopen standard defines them. PDO 1 TX is always configured at 0x1800, PDO 2 TX at 0x1801, PDO 1 RX at 0x1400, and so on. You see this column in Sheet 2 for transparency, not as something you fill in.

## Why can't I just use a float for my sensor value?

CANopen (CiA 301) does not define a standard REAL32 mapping for PDOs. While it is technically possible to pack 4 raw bytes of an IEEE 754 float into a PDO frame, both ends need custom code to interpret it correctly and it is not interoperable with off-the-shelf CANopen tools. The standard approach is to scale your value to an integer. For example, instead of sending 3.714 V as a REAL32, send 3714 as a UINT16 (millivolts) and document the scaling factor in the Description column.

## How do I know if my update rate will cause problems?

A 500 kbps CAN bus can theoretically carry around 4,000–5,000 short frames per second (a standard 8-byte frame takes about 130 µs including inter-frame spacing). In practice, budget for roughly 2,000 usable frames per second across all nodes combined to leave headroom for NMT, heartbeats, SDO traffic, and retransmissions. If your update rate would produce more than a few hundred frames per second from your node alone, talk to the software team before submitting.

## What if I don't know the exact update rate yet?

Write your best estimate and mark it TBD. It is better to give an approximate answer than to leave the column blank. The software team can work with 'approximately every 200 ms, TBD' but cannot do anything with an empty cell.

Questions?

Contact the Software or OBC subteam lead. Include your completed Sheet 4 rows and Node Registry entry so the conversation has context.